# ✅ FULL ASSESSMENT PACKAGE (BASED ON FULL CONTENT)

L.O 1: Perform Pre-Operation Activities
Topic: Interpretation of Power Plant Equipment Operation Manuals

# I. TEN (10) MULTIPLE-CHOICE QUESTIONS

1. Which of the following best defines a power plant?
A. A building used to store mechanical equipment
B. A facility that generates electricity for distribution
C. A system that only uses renewable resources
D. A station used for water purification
Answer: B

2. Which of the following is a renewable source of energy?
A. Coal
B. Natural gas
C. Solar energy
D. Petroleum
Answer: C

3. Which of the following is a major disadvantage of hydroelectric power plants?
A. They cause heavy smoke emissions
B. High capital cost due to dam construction
C. They require a large amount of fuel
D. They cannot start quickly
Answer: B

4. In a hydropower plant, the penstock is used to:
A. Filter impurities in water
B. Carry water from the reservoir to the turbine
C. Store water during dry seasons
D. Convert mechanical energy into electricity
Answer: B

5. Which turbine is commonly used for high-head hydropower plants?
A. Kaplan
B. Francis
C. Pelton wheel
D. Gas turbine
Answer: C

6. Diesel power plants are mostly used for:
A. Large-scale continuous power generation
B. Areas with abundant coal
C. Standby and emergency power supply
D. Long-distance electricity transmission
Answer: C

7. Which of the following is not an advantage of a diesel power plant?
A. Quick starting
B. High thermal efficiency
C. Low running cost
D. Can be installed anywhere
Answer: C

8. The main environmental disadvantage of steam power plants is:
A. Excessive water use
B. High smoke and fume emissions
C. Difficulty installing the plant
D. High construction cost
Answer: B

9. The device that converts mechanical energy into electrical energy in power plants is:
A. Turbine
B. Alternator
C. Generator
D. Boiler
Answer: B
(Alternator is the correct term in this context)

10. In a hydropower plant, the component that protects the penstock from excess pressure is:
A. Sluice valve
B. Surge tank
C. Spillway
D. Guide blade
Answer: B

# II. TEN (10) TRUE OR FALSE QUESTIONS

1. Renewable energy sources replenish naturally over a short period.
➡️ True

2. Nuclear power is considered a renewable source of energy.
➡️ False

3. Hydropower plants do not produce smoke during electricity generation.
➡️ True

4. Diesel power plants require large amounts of water for operation.
➡️ False

5. Steam power plants use the Rankine cycle to generate electricity.
➡️ True

6. Pelton wheel turbines are used for low-head applications.
➡️ False

7. Diesel power plants have high running costs due to expensive fuel.
➡️ True

8. Spillways are used to discharge excess water from the reservoir.
➡️ True

9. A surge tank stores fuel for the diesel power plant.
➡️ False

10. A hydroelectric power plant cannot be installed in hilly areas.
➡️ False

# III. FIVE (5) MATCHING QUESTIONS

Match the component in Column A with its correct description in Column B.

### COLUMN A

- Penstock

- Alternator

- Spillway

- Boiler

- Diesel Engine

### COLUMN B

A. Converts mechanical energy into electrical energy
B. Carries water under pressure to the turbine
C. Burns fuel to generate mechanical power
D. Releases excess water from the dam
E. Produces steam by burning coal

### ANSWERS:

1-B
2-A
3-D
4-E
5-C

# IV. FIVE (5) FILL-IN-THE-BLANK QUESTIONS

1. The _______ cycle is used in steam power plants to convert heat to mechanical energy.
➡️ Rankine

2. In a hydropower system, the _______ converts hydraulic energy into mechanical energy.
➡️ turbine

3. The device used to reduce pressure swings in the penstock is called a _______.
➡️ surge tank

4. Coal, oil, and natural gas are examples of _______ energy sources.
➡️ non-renewable

5. A _______ power plant uses diesel fuel to generate electricity.
➡️ diesel

# V. TEN (10) OPEN-ENDED QUESTIONS

1. Explain the difference between renewable and non-renewable energy sources, giving two examples of each.

2. Describe how a hydroelectric power plant converts stored water into electrical energy.

3. What factors should be considered when selecting a site for a hydroelectric power plant?

4. Discuss the role of the penstock and surge tank in hydropower plant operation and safety.

5. Compare Pelton, Francis, and Kaplan turbines in terms of head requirements and operating principles.

6. Explain the advantages and disadvantages of diesel power plants in remote or emergency applications.

7. Describe the main stages involved in the operation of a steam power plant.

8. What environmental impacts are associated with steam and diesel power plants, and how can they be minimized?

9. Identify the major auxiliary systems in a diesel power plant and explain their functions.

10. Discuss the technical, economic, and environmental factors that influence the choice of a power plant type for a specific region.

# ✅ I. TEN (10) MULTIPLE-CHOICE QUESTIONS

1. In a modern thermal power station operating at 50% load factor, burning 20,000 tons of coal per month, the ash produced is approximately:
A. 1–2%
B. 5–10%
C. 10–15%
D. 20–30%
Answer: C

2. The primary purpose of a superheater in a steam power plant is to:
A. Increase boiler water circulation
B. Convert wet steam into superheated steam
C. Cool flue gases before exiting the chimney
D. Lower fuel consumption in the furnace
Answer: B

3. In a hydropower plant, an economizer is used to:
A. Raise turbine speed
B. Heat feed water using flue gases
C. Convert heat into mechanical energy
D. Remove impurities from steam
Answer: B

4. The device that increases air temperature for better coal combustion is the:
A. Steam turbine
B. Air preheater
C. Boiler drum
D. Feed pump
Answer: B

5. Which type of condenser mixes cooling water directly with exhaust steam?
A. Surface condenser
B. Jet condenser
C. Regenerative condenser
D. Vacuum condenser
Answer: B

6. Which type of boiler is most widely used in large steam power stations?
A. Fire tube boiler
B. Water tube boiler
C. Horizontal tubular boiler
D. Locomotive boiler
Answer: B

7. In a nuclear reactor, the function of the moderator is to:
A. Absorb excess neutrons
B. Increase the temperature of coolant
C. Slow down fast-moving neutrons
D. Convert heat into steam
Answer: C

8. In a nuclear power plant, the control rods are made of:
A. Lead
B. Sodium
C. Cadmium
D. Thorium
Answer: C

9. In a gas turbine plant, the starting motor is used to:
A. Rotate the turbine at full speed
B. Start the compressor before turbine operation
C. Preheat the combustion chamber
D. Remove exhaust gases
Answer: B

10. The overall efficiency of a typical steam power station is about:
A. 10%
B. 20%
C. 29–30%
D. 50%
Answer: C

# ✅ II. TEN (10) TRUE OR FALSE QUESTIONS

1. More than 50% of the heat from coal combustion is lost in the condenser of a steam power plant.
✔ True

2. Superheated steam reduces blade corrosion in turbines.
✔ True

3. A surface condenser mixes cooling water and steam directly.
✘ False (That is a jet condenser.)

4. A nuclear power plant requires large amounts of coal supply.
✘ False

5. Gas turbine power plants require very large amounts of water for their condensers.
✘ False (They do not use condensers.)

6. Air pre-heaters help improve thermal efficiency.
✔ True

7. Water tube boilers can work at higher pressures than fire tube boilers.
✔ True

8. Uranium-235 releases less energy than burning coal.
✘ False (1 kg U-235 = 4,500 tons of coal.)

9. Gas turbines are difficult to start because the compressor needs power initially.
✔ True

10. Nuclear power plants are highly suited for rapid load changes.
✘ False

# ✅ III. FIVE (5) MATCHING QUESTIONS

Match Column A (components) with Column B (functions):

### COLUMN A

- Superheater

- Economizer

- Moderator

- Regenerator

- Boiler furnace

### COLUMN B

A. Supplies heat to air using exhaust gases
B. Slows down neutrons in a nuclear reactor
C. Burns fuel to generate heat
D. Heats feedwater using flue gases
E. Increases steam temperature above saturation point

### ANSWERS:

1–E
2–D
3–B
4–A
5–C

# ✅ IV. FIVE (5) FILL-IN-THE-BLANK QUESTIONS

1. The ratio of heat converted into mechanical energy to the heat supplied by fuel is called ______ efficiency.
➡ thermal

2. In a water tube boiler, water flows inside the ______ and hot gases flow outside.
➡ tubes

3. The major neutron absorber used in control rods of a nuclear reactor is ______.
➡ cadmium

4. A ______ condenser does not mix cooling water with exhaust steam.
➡ surface

5. In a gas turbine plant, heat is added to compressed air in the ______ chamber.
➡ combustion

# ✅ V. TEN (10) OPEN-ENDED QUESTIONS

- Explain why superheating steam improves the efficiency and safety of a steam power plant.

- Describe the flow path of flue gases through boiler components, from furnace to chimney, and explain the purpose of each component.

- Compare water tube and fire tube boilers in terms of construction, operating pressure, and advantages.

- Discuss the advantages and disadvantages of using jet condensers versus surface condensers.

- Explain the working principle of a nuclear reactor and the role of fuel rods, moderator, coolant, and control rods.

- What are the major factors that must be considered when selecting a site for a nuclear power station?

- Describe, step by step, how energy is converted from nuclear fission into electrical energy in a nuclear power plant.

- Explain the main components of a gas turbine power plant and describe the function of each.

- Why is the overall efficiency of a steam power plant relatively low despite improvements in components like economizers and air pre-heaters?

- Compare steam, nuclear, and gas turbine power plants in terms of efficiency, cost, size, environmental impact, and suitability for different applications.