ELTAI501 – ELECTRICAL AUTOMATION SYSTEM INSTALLATION`

DATE: …/…/2025

PERIOD: ………………….

ELTAI501 – ELECTRICAL AUTOMATION SYSTEM INSTALLATION`

DATE: …/…/2025

PERIOD: ………………….

TECHNICAL SECONDARY SCHOOLS (TSS)

END OF TERM I EXAMINATIONS: 2025-2026

MARKING GUIDES

(ANSWER WITH MARKS ALLOCATIONS)

SECTOR: Energy

TRADE NAME: Electrical Technology

RQF LEVEL: 5

MODULE TITLE: ELECTRICAL AUTOMATION SYSTEM INSTALLATION

…………/100

…………/100

MARKS:

DURATION: 3 HOURS

This Exam paper is composed of Twenty-four (24) questions distributed in Three Sections (A, B, C). Follow the instructions given below, and answer the indicated questions in each section for a total of 100marks

Section A: Seventeen (17) questions, all are Compulsory.	     55 marks

Section B: Five (5) questions, choose and answer any three (3).     30 marks

Section C: Two (2) questions, choose and answer any one (1)       15 marks

Allowed Materials: Blue pen, Ruler, Calculator, Pencil, (Fitting with the exam paper)

This Exam paper is composed of Twenty-four (24) questions distributed in Three Sections (A, B, C). Follow the instructions given below, and answer the indicated questions in each section for a total of 100marks

Section A: Seventeen (17) questions, all are Compulsory.	     55 marks

Section B: Five (5) questions, choose and answer any three (3).     30 marks

Section C: Two (2) questions, choose and answer any one (1)       15 marks

Allowed Materials: Blue pen, Ruler, Calculator, Pencil, (Fitting with the exam paper)

INSTRUCTIONS TO CANDIDATES:

|  | Which of the following is the primary advantage of integrating a Distributed Control System (DCS) with an automation system? A) Reduced power consumption B) Centralized control with high redundancy C) Decentralized control with high system reliability D) Reduced system complexity and lower maintenance cost Answer Answer: C) Decentralized control with high system reliability | (2Marks) |
| --- | --- | --- |
|  | 2. When designing the control circuit for an automation system, which of the following is the most critical factor for ensuring reliable communication between components? A) Control signal power supply consistency B) Proper configuration of communication protocols like Modbus RTU or TCP/IP C) Frequency of data polling D) The physical distance between PLC and I/O modules Answer: B) Proper configuration of communication protocols like Modbus RTU or TCP/IP | (2Marks) |
|  | ……3. In an automation system, which of the following best describes the function of the Human-Machine Interface (HMI)? A) To control and store process data B) To provide a real-time graphical representation for monitoring and controlling machine processes C) To monitor network communication between PLC and sensors D) To convert control signals into physical actions in actuators Answer: B) To provide a real-time graphical representation for monitoring and controlling machine processes | (2Marks) |
|  | 4. Which of the following data security measures should be prioritized when managing sensitive data in an automation system? A) High-speed data transfer protocols B) Implementation of encryption for data in transit and at rest C) Regular software updates for devices D) Minimizing system downtime for optimal performance Answer: B) Implementation of encryption for data in transit and at rest | (2Marks) |
|  | 5. When selecting Programmable Logic Controllers (PLCs) for an automation system, which of the following characteristics should be considered to ensure compatibility with the specific application needs? A) Processing speed and memory capacity B) User interface options and aesthetics C) The brand and model of the communication cables D) Size and weight of the PLC enclosure Answer: A) Processing speed and memory capacity | (2Marks) |
|  | 6. What is the primary function of the output modules in a PLC-based automation system? A) To convert digital signals into analog signals B) To send control signals to actuators and devices in the field C) To monitor the status of sensors in real-time D) To store system configuration and logic programs Answer: B) To send control signals to actuators and devices in the field | (2Marks) |
|  | 7. In the context of flexible automation, which of the following is the key advantage that allows rapid adaptation to different production processes? A) High capital investment required B) Automation system is designed for a specific product C) Machines and equipment can be easily reconfigured for different tasks D) Limited human involvement in decision-making Answer: C) Machines and equipment can be easily reconfigured for different tasks | (2Marks) |
|  | 8. Which of the following is the most suitable type of PLC for complex, multi-stage manufacturing processes that require extensive I/O configurations? A) Fixed (Compact) PLC B) Modular PLC C) Distributed PLC D) Remote PLC Answer: B) Modular PLC | (2Marks) |
|  | 9. During the data collection process in an automation system, which technique is most effective for gathering insights directly from system operators and maintenance staff? A) Surveys B) Observation C) Interviews D) Focus Groups Answer: C) Interviews | (2Marks) |
|  | When configuring data transmission settings for a PLC in an automation system, which of the following configurations is critical for ensuring proper communication with external devices? A) Device Addressing and Mapping B) Device ID and Access Control C) User Interface Configuration D) File Format Compatibility Answer: A) Device Addressing and Mapping | (2Marks) |
|  | 1. The HMI (Human-Machine Interface) is only used for controlling an automation system and does not play a role in monitoring the system's performance. TRUE OR FALSE Answer: False (Explanation: HMI is used for both controlling and monitoring the system's performance, providing real-time data and allowing operators to interact with the system.) | (3Marks) |
|  | 2. In a flexible automation system, the configuration of the machinery and equipment can be easily changed to produce different products with minimal downtime. TRUE OR FALSE Answer: True (Explanation: Flexible automation systems are designed to quickly adapt to different product configurations, enabling efficient changeovers with minimal downtime.) | (3Marks) |
|  | . A modular PLC system cannot be expanded or modified once it is installed, as it is designed to be a fixed configuration. TRUE OR FALSE Answer: False (Explanation: Modular PLC systems are designed to be flexible and can be easily expanded or modified by adding or removing modules as needed.) | (3Marks) |
|  | The communication protocol Modbus RTU is typically used for communication over Ethernet, allowing multiple devices to be connected in a network. TRUE OR FALSE Answer: False (Explanation: Modbus RTU is used for serial communication, typically over RS-485 or RS-232, while Modbus TCP/IP is the variant used for communication over Ethernet.) | (3Marks) |
|  | In an automation system, data from sensors is primarily used for input to the PLC, which then processes the data and sends output signals to actuators and other components. TRUE OR FALSE Answer: True (Explanation: Sensors provide input data to the PLC, which processes the data and controls actuators or other system components based on the results.) | (3Marks) |
|  | - language. Question 3: Match the types of automation systems with their applications: A – Automation System Types; B – Applications. 1. Fixed Automation; a. Used for tasks requiring frequent changes in processes or product types.. 2. Programmable Automation; b. Suitable for high-volume, repetitive tasks where the production process is standardized.. 3. Flexible Automation; c. Used for tasks that require automated changes to production based on different product designs.. 4. Integrated Automation; d. Incorporates systems like SCADA, PLCs, and HMIs to manage and optimize manufacturing processes in real-time.. 5. Supervisory Control and Data Acquisition (SCADA); e. Combines hardware and software components to monitor and control complex industrial processes.. Answer - Fixed Automation → b. Suitable for high-volume, repetitive tasks where the production process is standardized. - Programmable Automation → a. Used for tasks requiring frequent changes in processes or product types. - Flexible Automation → c. Used for tasks that require automated changes to production based on different product designs. - Integrated Automation → d. Incorporates systems like SCADA, PLCs, and HMIs to manage and optimize manufacturing processes in real-time. - Supervisory Control and Data Acquisition (SCADA) → e. Combines hardware and software components to monitor and control complex industrial processes. | (10Marks) |
|  | Question 2: Match the PLC programming languages with their characteristics: A – PLC Programming Languages; B – Characteristics. 1. Ladder Logic (LAD); a. Structured text-based language with high-level programming constructs like loops and conditional statements.. 2. Function Block Diagram (FBD); b. Graphical programming language that uses function blocks to represent the logic.. 3. Structured Text (ST); c. Graphical programming language resembling electrical relay logic.. 4. Sequential Function Chart (SFC); d. Ideal for processes with complex sequential operations, uses steps and transitions.. 5. Instruction List (IL); e. A low-level textual programming language similar to assembly language.. Answer Key: - Ladder Logic (LAD) → c. Graphical programming language resembling electrical relay logic. - Function Block Diagram (FBD) → b. Graphical programming language that uses function blocks to represent the logic. - Structured Text (ST) → a. Structured text-based language with high-level programming constructs like loops and conditional statements. - Sequential Function Chart (SFC) → d. Ideal for processes with complex sequential operations, uses steps and transitions. - Instruction List (IL) → e. A low-level textual programming language similar to assembly language. | (10Marks) |

|  | Explain the critical advantages and disadvantages of implementing an automation system in industrial applications. How can these factors influence decision-making? Answer: Advantages: - Increased Efficiency: Automation speeds up processes by reducing manual intervention, enhancing production rates. - Accuracy and Consistency: Automated systems minimize human error, ensuring uniform product quality. - Reduced Labor Costs: Once installed, automation systems reduce the need for manual labor, lowering operational costs. - Enhanced Safety: Dangerous tasks can be automated, improving worker safety and reducing workplace injuries. - Scalability: Automation allows for easy system expansion without significant increases in labor. Disadvantages: - High Initial Cost: The purchase, installation, and maintenance of automation systems require significant capital. - System Complexity: Implementing and troubleshooting automation systems can be complex and requires skilled personnel. - Limited Flexibility: Fixed automation systems cannot easily adapt to changes in product design. - Data Security Risks: Automation systems connected to networks may be vulnerable to cyber-attacks and data breaches. - Job Displacement: Automation may lead to the displacement of workers, requiring retraining for new roles. Influence on Decision-Making: - Cost-benefit analysis helps determine if automation enhances long-term profitability. - Industries with hazardous environments prioritize automation for safety. Dynamic production lines may favor flexible automation systems despite higher costs. | (10marks) |
| --- | --- | --- |
|  | What are the key preparation steps when conducting an automation system site visit, and why is each step crucial? Answer: - Gather Information: Understand project scope, site conditions, and existing systems to anticipate challenges. - Prepare Questions: Formulate questions on power sources, communication protocols, and equipment layout for clarification. - Logistics Planning: Arrange necessary tools, protective gear, and personnel to ensure efficient data collection. - Automation-Related Data Review: Analyze data types (sensor, machine, process) to assess system compatibility and security risks. - Safety Compliance Check: Verify adherence to safety standards to mitigate risks during the site visit. These steps ensure a systematic evaluation, facilitate smooth integration of the automation system, and prevent oversights. | (10marks) |
|  | Compare and contrast fixed, programmable, flexible, and integrated automation systems in terms of structure, functionality, and application. Answer: - Fixed Automation: - Structure: Hardwired, dedicated machinery. - Functionality: Performs repetitive tasks with little variation. - Application: High-volume production lines (e.g., automotive manufacturing). - Programmable Automation: - Structure: Configurable software and hardware. - Functionality: Allows reprogramming for different tasks. - Application: Batch production (e.g., pharmaceutical packaging). - Flexible Automation: - Structure: Modular systems with adaptive controls. - Functionality: Quick adaptation to new processes. - Application: Customizable manufacturing (e.g., consumer electronics). - Integrated Automation: - Structure: Fully interconnected systems with central monitoring. - Functionality: Synchronizes entire production processes. - Application: Large-scale industrial plants (e.g., power generation). | (10marks) |
|  | Discuss the importance of drawing standards in automation systems and how they ensure design consistency and operational efficiency. Answer: Drawing standards define conventions for presenting automation schematics. They ensure: - Clarity: Consistent symbols (IEC, ANSI) allow engineers worldwide to interpret drawings accurately. - Error Reduction: Standardized layouts minimize miscommunication and installation errors. - Compliance: Adherence to regulations (ISO 1219 for fluid systems) ensures legal compliance and safety. - Efficiency: Facilitates faster troubleshooting and maintenance. | (10marks) |
|  | . What are the essential components of a power and control circuit in an automation system, and how does each function? Answer: - Power Source: Provides electrical energy (AC/DC) to the system. - PLC (Programmable Logic Controller): Central unit controlling operations via user-defined logic. - HMI (Human-Machine Interface): Facilitates user interaction with system parameters. - Inputs: Sensors and switches feed data into the PLC. - Outputs: Actuators and relays perform physical tasks. - Communication Protocol: Modbus TCP/IP or RTU for real-time data exchange. | (10marks) |

|  | Describe the steps involved in assembling and wiring automation system components. Why is each step critical? Answer: - Preparation: Verify parts and tools against the BoM. - Component Mounting: Secure components using DIN rails. - Wiring Management: Route cables neatly to prevent interference. - Connection: Follow wiring diagrams for PLC and I/O modules. - Power Supply Installation: Ensure proper grounding and surge protection. - Testing and Validation: Confirm functionality through systematic checks. These steps prevent errors, ensure safety, and facilitate maintenance. | (15marks) |
| --- | --- | --- |
|  | What are the key factors when selecting and configuring a PLC for an automation project? Answer: - PLC Type: Choose fixed (compact) for small tasks, modular for complex systems. - I/O Requirements: Evaluate the number of input/output channels needed. - Communication Protocol: Ensure compatibility (Modbus, Profibus). - Processing Speed: Match PLC speed with system cycle time. - Scalability: Ensure the PLC can accommodate future expansions. - Environmental Conditions: Consider temperature, humidity, and electromagnetic interference. | (15marks) |

______END OF ASSESSMENT_________