SECTION A

MULTIPLE CHOICE

1. Pneumatics primarily uses which medium for power transmission?
A. Water
B. Oil
C. Compressed air
D. Steam
Answer: C

2. Which of the following is NOT a primary function of a hydraulic reservoir?
A. Dissipating heat
B. Air separation
C. Fluid compression
D. Contamination settling
Answer: C

3. A 4/3 valve has:
A. 4 ports, 3 positions
B. 3 ports, 4 positions
C. 4 paths, 3 actuators
D. 3 ports, 4 switching signals
Answer: A

4. A hydraulic hose must be selected based on:
A. Colour and appearance
B. Pressure rating, temperature, fluid compatibility
C. The brand name
D. Weight only
Answer: B

5. Which pump type delivers constant flow regardless of pressure?
A. Centrifugal pump
B. Gear pump
C. Rotodynamic pump
D. Dynamic pump
Answer: B (a positive displacement pump)

6. A flow control valve is used to:
A. Change fluid type
B. Control actuator speed
C. Store hydraulic fluid
D. Increase compressor horsepower
Answer: B

7. Which fitting is commonly used for quick assembly without tools?
A. Threaded fitting
B. Barbed fitting
C. Push-to-connect fitting
D. Compression fitting
Answer: C

8. In indirect control, the actuator movement is triggered by:
A. Direct push button
B. Manual valve
C. Relay or solenoid control
D. Mechanical steering
Answer: C

9. A flow sensor that uses heat transfer to measure flow is known as:
A. Electromagnetic
B. Thermal mass
C. Turbine
D. Ultrasonic
Answer: B

10. Which device stores pressurized fluid energy in a hydraulic system?
A. Pump
B. Accumulator
C. Cylinder
D. Motor
Answer: B

# TRUE OR FALSE QUESTIONS

1. Hydraulics uses a compressible fluid to transmit power.
False (Hydraulics uses non-compressible liquids)

2. Pneumatic systems usually operate at lower pressures than hydraulic systems.
True

3. Filters in hydraulic systems remove contaminants to protect components.
True

4. A double-acting cylinder has only one port.
False

5. Cavitation is a common cause of pump failure.
True

6. All hose fittings in pneumatic systems require crimping.
False (e.g., push-to-connect require no crimping)

7. Flow control valves are used to regulate actuator speed.
True

8. Hydraulic flanges are not suitable for high-pressure applications.
False

9. Workplace ergonomics includes the correct use of furniture and layout design.
True

10. Indirect control uses electrical or logic elements to control pneumatic actuators.
True

# ✅ 5 FILL IN THE BLANK QUESTIONS

1. The device used to measure fluid pressure in hydraulic and pneumatic systems is called a ____________.
Answer: Pressure gauge

2. A ____________ is used to reduce and maintain a set output pressure in a circuit.
Answer: Pressure-reducing valve

3. Pneumatic actuators convert compressed air energy into ____________ or ____________ motion.
Answer: Linear / rotary

4. A ____________ pump traps a fixed volume of fluid and forces it through the discharge line.
Answer: Positive displacement

5. Threaded connections require ____________ tape or sealant to prevent leaks.
Answer: Teflon (or thread sealant)

# MATCHING QUESTIONS

### Match COLUMN A with COLUMN B

| COLUMN A | COLUMN B |
| --- | --- |
| 1. Accumulator | A. Stores pressurized fluid energy |
| 2. Compressor | B. Converts mechanical power into air pressure |
| 3. Flow sensor | C. Measures rate of fluid movement |
| 4. Reservoir | D. Stores and conditions hydraulic fluid |
| 5. Coupling | E. Connects two shafts for power transmission |

Answers:
1 – A
2 – B
3 – C
4 – D
5 – E

## SECTION B. Short Answer (5 Questions)

- Describe what the arrows inside a DCV symbol indicate.

- What does a pilot line in pneumatic symbols represent?

- Explain the graphical difference between a fixed and adjustable flow control valve.

- Draw or describe the symbol for a pressure relief valve.

- Why do hydraulic symbols not show internal construction details?

## SECTION C. Open-Ended (2 Questions)

- Explain how to interpret a 4/3 hydraulic directional control valve symbol, including ports, positions, and internal arrows.

- Discuss the importance of standardization of hydraulic and pneumatic symbols in industry.