# SECTION A — MULTIPLE CHOICE (10 questions × 2 marks = 20 marks)

Choose the correct answer.

- Electrical conduit is used for:
a) Carrying water
b) Protecting and routing electrical wiring
c) Measuring voltage
d) Supporting lighting fixtures

- AC current is defined as:
a) Current flowing only in one direction
b) Current that periodically reverses direction
c) Current that flows through batteries
d) Current used only in communication systems

- A junction box is used to:
a) Control ON/OFF operations
b) House electrical connections
c) Secure cable clips
d) Measure current

- A wiring diagram shows:
a) Physical layout of installation
b) Electrical connection using symbols
c) Location of fixtures in a building
d) Specifications of materials

- The size of a conductor is expressed in:
a) Ohms
b) mm² cross-sectional area
c) Volts
d) Watts

- A PVC bendit is used to:
a) Cut conduit
b) Heat and soften PVC pipe for bending
c) Test continuity
d) Support trunking

- Non-metallic (NM) sheathed wires are used for:
a) Outdoor lighting
b) In-house wiring
c) High-voltage transmission
d) Energy meter connections only

- A ring terminal is best described as:
a) A connector shaped like a fork
b) A fully round terminal used with a screw
c) A connector used for coax cables
d) A switch terminal

- Surface-mounted installation is:
a) Installed inside the wall
b) Installed outside the wall
c) Installed underground
d) Installed on a metallic roof

- A pipe bender is used for:
a) Stripping wires
b) Twisting cables
c) Bending conduit to shape
d) Hammering fittings

# SECTION B — TRUE OR FALSE (10 questions × 1 mark = 10 marks)

- DC current flows only in one direction. ______

- Cleaning means removing unwanted materials. ______

- A switch box houses electrical connections. ______

- Trunking is always circular in shape. ______

- Built installation is performed inside walls. ______

- A coaxial cable is used for high-frequency signal transmission. ______

- A coupling is used to join two conduits. ______

- Pipe curves are always 90°. ______

- Wire strippers are used for removing insulation from conductors. ______

- A socket outlet must be installed at least 450 mm above the floor. ______

# SECTION C — FILL IN THE BLANKS (5 questions × 2 marks = 10 marks)

- The ______ value is the maximum value of an alternating quantity.

- A ______ is a tool used for measuring distances when installing conduits.

- The document used in tendering that itemizes materials and costs is called a ______.

- A ______ is a type of terminal shaped like a fork.

- The process of changing conduit direction without accessories is called conduit ______.

# SECTION D — MATCHING (5 questions × 2 marks = 10 marks)

Match column A with column B.

### Column A

A. Electrical switch
B. Elbow 90°
C. Insulated conductor
D. Architectural diagram
E. Side cutter pliers

### Column B

- Used to cut and form loops on large conductors

- Shows the location of each component in a building

- Used to change conduit direction at a right angle

- Interrupts or changes current flow

- Wire covered with insulation

Write answers like: 26. A–4

- A – ___

- B – ___

- C – ___

- D – ___

- E – ___

# SECTION E — OPEN-ENDED QUESTIONS (10 questions × 4 marks = 40 marks)

- Explain the parameters to consider when locating a power source in an installation site.

- Describe the difference between built installation and surface-mounted installation.

- List and explain three types of electrical diagrams.

- Explain the procedure for chiselling during conduit placement.

- Differentiate between PVC pipe and trunking.

- Describe the steps involved in measuring and cutting conduits.

- Explain any four factors that affect cable current-carrying capacity.

- Describe the tools used in connecting electrical wires and their uses.

- Explain the difference between ring terminal, spade terminal, and bullet terminal.

- Describe the procedure for fixing and testing a lighting fixture.

# SECTION A — MULTIPLE CHOICE (10 × 2 = 20 marks)

- b

- b

- b

- a

- b

- b

- b

- b

- b

- c

# SECTION B — TRUE / FALSE (10 × 1 = 10 marks)

- True

- True

- False (A switch box houses switch mechanisms, NOT wire connections)

- False (Trunking is rectangular/square)

- True

- True

- True

- False (Curves may be 45° or 90°)

- True

- True

# SECTION C — FILL IN THE BLANKS (5 × 2 = 10 marks)

- Peak

- Measuring tape / tape measure

- Bill of quantities

- Spade terminal

- Shaping

# SECTION D — MATCHING (5 × 2 = 10 marks)

| No. | Column A | Correct Match |
| --- | --- | --- |
| 26 | A — Electrical switch | 4 |
| 27 | B — Elbow 90° | 3 |
| 28 | C — Insulated conductor | 5 |
| 29 | D — Architectural diagram | 2 |
| 30 | E — Side cutter pliers | 1 |

# SECTION E — OPEN-ENDED QUESTIONS (10 × 4 = 40 marks)

### 31. Parameters for locating a power source

- It must be located outside the structure for safe access.

- Must be in an area free from excess moisture, water, or humidity.

- Must be placed where there is minimal vibration.

- Should be protected from excessive heat.

- Should allow easy maintenance and safe isolation.

### 32. Difference between built installation and surface-mounted installation

- Built installation:

  - Performed inside the wall.

  - Uses PVC or metal conduits buried inside plaster.

  - More aesthetic and protected.

- Surface-mounted installation:

  - Installed outside the wall using trunking or exposed conduits.

  - Easier to maintain and modify.

  - Common in already finished buildings.

### 33. Three types of electrical diagrams

- Schematic diagram: Shows electrical components and connections using symbols.

- Circuit diagram: Simplified representation focusing on component connection paths.

- Architectural diagram: Shows the physical location of electrical components in a building.

- Wiring diagram (optional): Shows the physical layout of wires and installation.

### 34. Procedure for chiselling during conduit placement

- Measure and mark conduit and fitting positions.

- Use chisel and hammer to cut along the marked line.

- Face the bevel down while chiselling.

- Remove thin slices gradually.

- Control depth by adjusting handle angle.

- Use grinding machine to clean edges if needed.

### 35. Difference between PVC pipe and trunking

- PVC pipe:

  - Circular conduit.

  - Used to protect wires inside walls.

  - Flexible and commonly used in domestic installation.

- Trunking:

  - Rectangular or square enclosure.

  - Mounted on wall surface.

  - Has a removable cover for easy maintenance.

### 36. Steps in measuring and cutting conduits

- Measure required conduit length.

- Mark measurement on pipe.

- Calculate extra length needed to compensate for bends.

- Cut using hacksaw or pipe cutter.

- Remove burrs using knife or reamer.

- Fit and test the conduit before installation.

### 37. Factors affecting cable current-carrying capacity

- Conductor size: Larger cross-section carries more current.

- Ambient temperature: Higher temperatures reduce capacity.

- Number of bundled conductors: More cables reduce heat dissipation.

- Insulation type: Some insulation materials tolerate more heat.

- Installation method: Buried cables carry less than open-air cables.

### 38. Tools used when connecting electrical wires

- Wire strippers: Remove insulation.

- Long-nose pliers: Form loops on conductors.

- Side cutters: Cut large cables.

- Screwdrivers: Tighten terminals.

- Electrician’s knife: Strip large cables.

- Measuring tape: Measure cable lengths.

### 39. Difference between ring, spade, and bullet terminals

- Ring terminal: Fully closed circle; used where a screw must pass through.

- Spade terminal: Fork shape; easy to insert/remove under a screw.

- Bullet terminal: Male–female connectors; quick to connect/disconnect.

### 40. Procedure for fixing and testing a lighting fixture

- Switch off power supply.

- Mark and drill holes for fixing points.

- Connect wires: live to live, neutral to neutral, earth to earth.

- Secure the fixture to the ceiling or wall.

- Restore power.

- Test the lighting using the switch and verify proper operation.

# ✅ All answers are provided fully and accurately.

If you want, I can also produce a marking guide, rubric, or student version of the exam.

Top of Form

Attach

Search

Study

Voice

Bottom of Form

C